GYS102J全液压振动

GYS102J FULL HYDRAULIC SINGLE DRUM

VIBRATORY ROLLER

| 规格型号：GYS102J型振动压路机 |  |
| --- | --- |
| MODEL: GYS102J FULL HYDRAULIC SINGLE DRUM VIBRATORY ROLLER 主要性能特点：Main characteristic 适用于高等级公路、大坝、铁路、机场等大型工程基础的压实，可碾压砂土、砾石、碎石、岩石填方等非粘性材料及半粘性材料。。 Suitable for compaction of foundations for large-scale engineering projects such as expressways, dams, railways, and airports, capable of compacting non-cohesive materials and semi-cohesive materials such as sandy soil, gravel, crushed stone, and rock fill. 性能特点： 1、液压双轮驱动，两档无级变速，爬坡性能好； Hydraulic two-wheel drive, two-speed continuously variable speed, good climbing performance. 2、液压振动，双频双幅，电控起振； Hydraulic vibration, dual-frequency double amplitude, electronically controlled vibration. 3、液压转向，铰接式车架，操作轻便灵活； Hydraulic steering, articulated frame, light and flexible operation. 4、压路机专用驱动桥，驱动性能优越； Equipped with a special drive axle for road rollers, offering superior drive performance. 5、液压行走制动，电控盘刹停车制动； Hydraulic traveling brake, electronically controlled disc brake parking brake. - 后机罩可大角度开起，维护保养触手可得； - The rear cover can be opened at a large angle, making maintenance and servicing easily accessible.. 7、驱动和振动系统均配置进口国际知名品牌液压元件，性能可靠 Equipped with imported international famous brand hydraulic components, reliable performance. 8、标配驾驶室选配冷暖空调； Standard cab , A/C optional . |  |
| 主要技术参数：Main parameters |  |
| 1、整机重量 Operating weight | 10000 kg |
| 2、振动轮直径 Drum Diameter | 1450 mm |
| 3、压实宽度 Compaction width | 2100 mm |
| 4、静线载荷 Static line pressure | 255 N/cm |
| 5、振动频率 Vibratory frequency | 30/35 Hz |
| 7、理论振幅 Vibratory amplitude | 1.8/0.9 mm |
| 8、激振力 Centrifugal force | 250/160 kN |
| 9、行驶速度 Travelling speed | 0-5，0-9 km/h |
| 10、爬坡能力 Gradeability | 50% |
| 11、转弯半径 Min. turning radius | 6200 mm |
| 12、轴距 Wheel base | 3000 mm |
| 13、最小离地间隙 Min. ground clearance | 380 mm |
| - 14、外形尺寸（长×宽×高） - Dimension (L×W×H) | 5850×2250×3010mm |
| 15、发动机 Engine |  |
| 型 号 Model | WP4G110E220 |
| 功 率 Rated power | 82 kW |
| 转 速 Rated speed | 2200 r/min |
| 16、燃油箱 Fuel tank | 240 L |
| 17、液压油箱 Hydraulic oil tank | 190 L |

主要配置说明 Main configuration

| 序号 S/N | 名 称 ITEM | 规格型号 MODEL | 数量 NO. | 制造商 MANUFACTURER |
| --- | --- | --- | --- | --- |
| 1 | 柴油机 Engine | WP4G110E220 | 1 | 潍柴 WEICHAI |
| 2 | 振动泵 Vibratory pump | HPV055 | 1 | 林德 LINDE |
| 3 | 振动马达 Vibratory motor | HMF055 | 1 | 林德 LINDE |
| 4 | 驱动泵 Travelling pump | HPV075 | 1 | 林德 LINDE |
| 5 | 钢轮马达 Drum motor | HMF055 | 1 | 林德 LINDE |
| 6 | 驱动马达 Travelling motor | HMV055 | 1 | 林德 LINDE |